# Noise Monitor System Messages and Decibel Ranges

# System Messages for Noise Warnings

1. Low Warning

⚠️ Moderate Noise Detected
Current decibel levels are slightly elevated. Please be mindful of your surroundings.

2. High Warning

⚠️ High Noise Level Warning
Prolonged exposure at the current level may cause discomfort. Action is recommended to reduce sound output.

3. High Alert

❗ Noise Violation Alert
Noise levels have exceeded permitted limits/set thresholds. Corrective action is required immediately to comply with guidelines.

4. Critical Alert

❌ CRITICAL NOISE ALERT: DANGER LEVEL
Extreme decibel levels detected. This environment poses an immediate risk of hearing damage. Seek immediate protective equipment or evacuate the area now.

# Decibel (dB) Ranges for Alerts

The following A-weighted decibel (dBA) ranges are based on public health and occupational safety guidelines from organizations like the National Institute for Occupational Safety and Health (NIOSH) and the Environmental Protection Agency (EPA).

| Message Level | dB Range (dBA) | Context/Guideline |
| --- | --- | --- |
| 1. Low Warning | 70 - 85 dBA | The EPA recommends staying below 70 dB over a 24-hour period to prevent any hearing loss over a lifetime. In this range (e.g., vacuum cleaner, busy traffic), exposure for several hours may cause hearing fatigue but is generally safe for short durations. |
| 2. High Warning | 85 - 100 dBA | This is the start of the "danger zone" where prolonged exposure can cause permanent hearing damage. NIOSH recommends an 8-hour exposure limit of 85 dBA. The safe exposure time halves for every 3 dB increase above 85 dBA (e.g., 4 hours at 88 dBA, 15 minutes at 100 dBA). |
| 3. High Alert | 100 - 120 dBA | Sounds in this range (e.g., snowmobiles, rock concerts, ambulance sirens) are very loud and require immediate action to use hearing protection or limit exposure time to mere minutes or seconds. |
| 4. Critical Alert | 120+ dBA | This range is at or near the threshold of pain and can cause immediate hearing damage or injury in very short timeframes (seconds). Exposure to sounds like a jet engine at 100 feet or a jackhammer requires immediate evacuation or specialized protection. |